Safety and Work Protocol for RC2 Woollam Ellipsometer:

Only Trained and Authorized Users are Allowed to Operate the System !

- Use gloves when handling with the system to prevent contamination.

- Power on the ellipsometer in the following order: power unit, light source unit, and then the software (the computer password is” Woollam!”).

- Ensure that your sample is clean; otherwise, it may affect the measurements.

- Select an appropriate recipe for your sample; you are not allowed to modify any existing recipes in the software.

- The system software is intended for brief analyses. For in-depth analyses, a separate computer is available for remote connection.

- If a malfunction occurs, immediately stop work, report the issue to the tool owner.

- Power off the ellipsometer in the following order: light source unit, software, computer and then the power unit.

Additional Accessories

To use the temperature-controlled stage or focusing probes, please coordinate with the tool owner.

For any questions, call 79812/ 08-6479812 Tania

For any problems